Goal:

Generate synthetic wafer maps for demonstration, benchmarking and Software QA purposes

Parameters

Must Have

- Wafer diameter

  - 150 mm, 200 mm, 300 mm

- Edge Exclusion

  - From 1 mm to 10 mm

- Die size

  - From 1x1 mm to 25 x 35 mm

  - Aspect ratio between 1:2 and 2:1

    - Meaning, 6 x 3 mm and 3 x 6 mm are OK, but 12 x 3 mm is not

  - Set scribe street to 0.1 mm

    - Nice to have: variable scribe street from 0.05 mm (50 um) to 0.2 mm (200 um)

- Flat vs Notch

  - 150 mm wafers have flats

  - 200, 300 mm wafers have notches

  - This should auto-select

- Wafer orientation

  - Flat/notch in any orientation

    - Bottom, right, left, top

    - Only 90 degree increments

- Wafer Quantity

  - 25 wafer maximum lot

  - Smaller lots, any number 1 .. 24

    - Often in increments of 3, eg 3, 6, 12..

- Wafer numbers

  - Sequential 1..25

- Yield

  - Option for Yield directly

  - Option for Defect Density with a simple model

    - Y = e^(-AD) where Area (A) is in sq cm and Defect Density (D) is in defects/cm2

    - Note, there are more complex yield models but this is a synthetic data generator

    - Note, sometimes people put D in Defects/in2, again, this is a synthetic data generator. Ignore that option.

- Test insertions

  - CP1

    - CP = Circuit Probe = TSMC & Taiwan foundry name for wafer sort

    - CP1 at one temperature, CP2 at another, etc. Example, CP1 room, CP2 cold, CP3 hot

  - CP1 & CP2

  - CP1, CP2, CP3

  - CP1 uses Yield Model to create P/F

  - CP2 Yield = 90% .. 99.9% of CP1

    - Only CP1 Pass units can Pass at CP2

  - CP3 Yield = 90% .. 99.9% of CP2

    - Only CP2 Pass units can Pass at CP3

  - Note: It is technically possible that the yield at CP2 is worse than at CP1, but this case is rare and usually means something is messed up at the customer. Hence ignore.

- Number of bins

  - Hardbins: 16, 64, 256

  - Softbins: Hardbins *4, *16, *64

  - For now, ignore any mapping between softbins and hardbins

- Test items

  - Between 100 and 1 M in orders of magnitude

    - No need to generate 307 test items. Either 100 or 1000

  - Pass/Fail vs Parametric %

    - Default to 50/50

      - Nice to have: Selectable in 10% increments

    - Pass/Fail will have 0 (fail) or 1 (pass) as result

    - Parametric: use Random Number Generator (RNG) to create REAL value (0.0 ..1.0)

      - This will create Gaussian distributions, we may want to change that later, but good for now

      - Data details

        - Option for exponential distribution, eg value = 10^(RNG result)

        - Option for quantized data, eg 0.0, 0.2, 0.4, 0.6, 0.8

        - Option for positive & negative values eg -1.0 .. + 1.0

        - Option for scientific notation data eg X.XXe-YY .. X.XXe+YY

        - Option for real value with only one value

  - Test item name

    - Simple, like PARAM_[number]

      - Suggest number have leading 0s, such as PARAM_0001 not PARAM_1

    - Nice to have

      - Verbose, length = 31, 63, 127, 255, including _[number]

        - Obnoxious verbose where only differentiation = [number]

          - This is a UI checker, because differentiation is at the very end of the string

        - Less obnoxious verbose where the beginning of the string is also differentiated. It can be gibberish, but use 8 character chunks delimited by underscore “_”

- Yield Patterns

  - Steve’s list. Please edit to include here, I don’t want to recreate from memory.

  - Repeating bad die, aka “repeaters”

    - Means, one die, or more, per stepping field will always fail

    - More likely that die size is entered than stepping field config, so tool must autogenerate a stepping field?

    - Nice to have

      - Soft repeaters = repeating bad die with failure rate < 100%

        - Between 10% extra and 100% extra, selectable

        - Can be quantized in 10% increments if that’s easier

      - Striping = yield loss at one edge of a stepping field

        - Top, bottom, left or right

        - 100% fail or soft-repeater style

        - Striping is caused by “lens tilt” in lithography, I can explain later if you are interested.

Nice To Have

- Lot numbers

  - Each 25 wafers in the fab has a lot number

    - Example FYYWWSSSS

    - Where F is the fab designator, an Alphabetic character

    - YY is the last 2 digits of the Year

    - WW is the work week, between 1 and 53

      - Some years have 53 WWs

    - SSSS is a sequential lot number from 0000 to 9999

  - Partial lots, aka “splits” or “child lots” usually have suffixes

    - Example FYWWSSS.01, .02, etc

    - There are other notations but this is a good start

- Lot sequence

  - If you generate multiple fab lots worth of wafers, they should have different time stamps for sort start time

    - 1 lot per month

    - 1 lot per week

    - 1 lot per day

    - Multiple lots per day

  - This is to test our trend chart capability

- Test Time

  - Wafer sort test time is expressed in seconds/touchdown

  - Between 1 second and 600 seconds (10 minutes)

  - This is important for timestamp on the per-unit data, if you get to that detail

- Multi-site

  - Between 1 site and 16 sites tested simultaneously

  - This is known as multi-site or parallelism

  - For Gross Die per Wafer, maximum # sites

    - <200 GDPW, 1 site (GDPW = Gross Die Per Wafer)

    - 200..399, 2 site

    - 400..799, 4 site

    - 800..1599, 8 site

    - 1600 +, 16 site

  - This is imperfect but it’s roughly correct

  - Patterns of multi-site can be

    - Side by side

    - Top & bottom

    - Square or rectangle pattern, eg 2x2 or 2x4

    - Checkerboard pattern, eg 2x4 checkerboard which covers 4 units

      - This is done to avoid wire routing difficulty on the probe card PCB

    - Diagonal pattern, either to left or right

      - This is done to avoid wire routing difficulty on the probe card PCB

    - There are other patterns but that’s plenty for now

- Site-to-Site Yield Loss (S2S)

  - Within multi-site test fixtures, there are often yield variations

  - Good result = yield results are random across the multi-site fixture

  - Problem result = yield loss on one or more sites

  - Method suggested, but needs discussion

    - Actual Yield = Yield Model * S2S factor

    - S2S factor on a random site in a multi-site setup

    - S2Sfactor = 0.0 .. 1.0

    - Good results have S2S factor > 95% for all sites

- Repair

  - Placeholder for repair

  - General idea, memory defects can be repaired by use of spare memory elements, such as spare rows or spare columns

  - Total yield = Virgin good die (no repair needed) + Repaired good die

  - Yield analysis should be able to differentiate Good, Virgin and Repaired, and calculate ratios

  - Generating synthetic data for this requires more thought